Product Specification – Icing with Cornflour

Description

A fine white powder, free from abnormal odours and flavours and free from visible extraneous material.

Chemical/Physical

|  | Max | Typically | Method |
| --- | --- | --- | --- |
| Solution Colour (ICUMSA Units) |  | 21-32 | METHOD-WS-008 50% sugar solution, 0.45µm membrane filtration, absorbance at 420nm |
| Loss on Drying (%) | 0.4 | 0.1-0.3 | METHOD-WS-004 Oven for 3hr @ 105oC |
| Reducing Sugars (%) | 0.04 | 0.001-0.004 | METHOD-WS-010 Sugar solution heated with alkaline copper, EDTA titration using murexide indicator |
| S02 (mg/kg) | 6 | 1-4 | METHOD-WS-020 Alkaline solution with bleached rosaniline and formaldehyde, absorbance at 570nm |
| Particle Size > 106µm (%) > 150 µm (%) < 75 µm (%) Mean aperture (µm) | 8.0 | <1.0 >80 19 - 26 | Laser particle sizer |
| Corn Starch | 3.5 | 2.0 - 3.0 | METHOD-SP-070 Dissolve, filter, dry and weigh |
| Lead (mg/kg) | 0.1 | <0.05 | Atomic absorption |
| Arsenic (mg/kg) | 1.0 | <0.05 | Atomic absorption |
| Copper (mg/kg) | 1.0 | <0.05 | atomic absorption |

Microbiological

|  | Typically | Limit |
| --- | --- | --- |
| Mesophilic Bacteria (cfu per 10g) | <500 | <1200 |
| Yeasts (cfu er 10g) | <10 | <50 |
| Moulds (cfu per 10g) | <10 | <50 |

Nutritional Data

| Per 100g | Typically |
| --- | --- |
| Energy | 1690kJ (398kCals) |
| Fat | 0g |
| Of Which Saturates | 0g |
| Carbohydrate | 99g |
| Of Which Sugars | 95g |
| Protein | 0g |
| Salt | 0g |

Origin

Produced from Sugar Beet grown in the UK and starch produced from maize grown in the Europe.

Additional Ingredients

Cornflour Starch (Free flow ingredient)

Food Intolerance Data - Product does not contain the following:

| Wheat / Barley / Rye / Oats Gluten | Animal/ Animal Derivatives | Polyols |
| --- | --- | --- |
| Yeast / Yeast Products | Meat Products | BHA’s / BHT’s |
| Soya / Soya Derivatives | Milk Products | Azo Dyes |
| Seed / Seed Derivatives | Lactose | Antioxidants |
| Sesame/Sesame Derivatives | Other Dairy Products | Phenylalanine |
| Nuts / Nut Derivatives | Mustard/ Mustard Products | Glutamates |
| Peanuts/Peanut Derivatives | Egg / Egg Products | Benzoates |
| Celery/Celeriac | Salt | HVP / TVP |
| Lupin / Lupin Derivatives | Preservatives | GMO's |
| Seafood/Seafood Derivatives | Artificial Colours | SO2 above10mg/kg |
| Fish/Fish Derivatives | Natural Colours | Caffeine |
| Crustaceans/molluscs & Derivatives | Mineral Hydrocarbons |  |

Certification

This product has Kosher status (Pareve), but not for Passover.

This product is certified Halal. (copies of certificates can be supplied on request)

Declaration for use

Sugar (free flow agent need not be declared if not functional in finished products)

Storage

Cool, dry environment, below 65%RH and at 10-20oC avoiding temperature fluctuations in excess of 5oC in any 24h period. The product should not be subject to any draughts. Avoid direct light and sources of ignition. Do not store close to substances with strong odours.

Shelf-life

Up to 32 weeks from date of manufacture if good storage practice is observed Usable life has been assessed as the period of time under appropriate storage conditions where the product remains free flowing.  Product that has exceeded its ‘usable life’ will be fit for use provided its flowability is acceptable as no other quality parameters will be affected. Does not require a best before date (refer to regulation 7, Food labelling regulations amendment 1996).

Pallet Specification

All BS pallets are wooden pallets, dimensions 1000mm x 1200mm (2.5), height 161mm (3) with nominal weight of 26kg. Open boarded two-way entry. Waterproof cover applied to deck before loading with bagged product.

Packaging

| Code | Description | Details | Dimensions |
| --- | --- | --- | --- |
| 55440 | 25kg Sack | Gusseted 3-ply paper sack (inc. LDPE coated layer) with stitched top and glued bottom. 40 sacks per pallet covered with polyethylene hood | Face width: 455 +/- 3mm Flat length: 790 +/- 10mm Gusset width: 75 +/- 3mm |
| 55452 | 1 Tonne | Woven polypropylene IBC, with polypropylene coating. Loops for four point lifting. Filling spout sealed with blue nylon MultiLoc security seal. Covered by polyethylene pallet hood | 88 x 96 x 160 cm 4 lifting loops |